Annex

A.	Proposed New Air Quality Objectives (AQOs)

| Pollutant | Averaging Time | Existing AQOs (µg/m3) | Proposed New AQOs vis-à-vis WHO AQGs / ITs (µg/m3) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | IT-1 | IT-2 | IT-3 | AQGs |
| SO2 | 10-minute | --- | --- |  |  | 500 |
|  | 24-hour | 350 | 125 | 50 |  | 20 |
| RSP (or PM10) | 24-hour | 180 | 150 | 100 | 75 | 50 |
|  | Annual | 55 | 70 | 50 | 30 | 20 |
| FSP (or PM2.5) | 24-hour | --- | 75 | 50 | 37.5 | 25 |
|  | Annual | --- | 35 | 25 | 15 | 10 |
| NO2 | 1-hour | 300 | --- |  |  | 200 |
|  | Annual | 80 | --- |  |  | 40 |
| O3 | 8-hour | 240 (1-hour) | 160 |  |  | 100 |
| CO | 15-minute | --- | --- |  |  | 100,000 |
|  | 30-minute | --- | --- |  |  | 60,000 |
|  | 1-hour | 30,000 | --- |  |  | 30,000 |
|  | 8-hour | 10,000 | --- |  |  | 10,000 |
| Pb | Annual | 1.5 (3-month) | --- |  |  | 0.5 |

Proposed new AQOs

B. Proposed Emission Control Measures

Emission Capping and Control

- Increasing the ratio of natural gas in local electricity generation to 50% with additional emission abatement measures

- Early retirement of aged / heavily polluting vehicles

- Earlier replacement of Euro III commercial diesel vehicles with models meeting latest Euro standards

- Wider use of hybrid / electric vehicles or other environment-friendly vehicles with similar performance

- Ultra low sulphur diesel for local vessels

- Selective catalytic reduction for local vessels

- Electrification of aviation ground support equipment

- Emission control for off-road vehicles / equipment

- Strengthening volatile organic compounds control

Transport Management

- Low emission zones

- Car-free zone / pedestrianisation scheme

- Bus route rationalization

Infrastructure Development and Planning

- Expand rail network

- Cycling network to major public transport hubs

Energy Efficiency Measures

- Mandatory implementation of Building Energy Codes

- Energy efficiency standards for domestic electrical appliances

- Light-emitting diode or equivalent alternatives for traffic signal / street lighting

- Tree planting / roof-top greening

- District cooling system for Kai Tak Development